#include <SoftwareSerial.h>

#include <Wire.h>

#include <LiquidCrystal_I2C.h>

// ============================================================

//  PIN DEFINITIONS

// ============================================================

#define RELAY_PIN       7

#define BUZZER_PIN      6

#define LED_GREEN       5

#define LED_RED         4

#define RDM_RX          2

#define RDM_TX          3

#define ROCKER_SWITCH   8

// ============================================================

//  RELAY CONFIGURATION – CHANGE THIS LINE IF NEEDED

// ============================================================

// Most relay modules: HIGH = relay ON (solenoid unlocked)

// Some modules:        LOW  = relay ON (solenoid unlocked)

// Try HIGH first. If solenoid opens when it should stay locked,

// change to LOW.

#define RELAY_UNLOCK_STATE HIGH

#define RELAY_LOCK_STATE   !RELAY_UNLOCK_STATE

// ============================================================

//  OBJECTS

// ============================================================

SoftwareSerial rdmSerial(RDM_RX, RDM_TX);

LiquidCrystal_I2C lcd(0x27, 16, 2);

// ============================================================

//  CONSTANTS

// ============================================================

#define SCAN_COOLDOWN   2000     // wait 2 seconds between card reads

#define UNLOCK_TIME     5000     // solenoid opens for 5 seconds

#define RDM_STX         0x02

#define RDM_ETX         0x03

#define RDM_PACKET_LEN  14

// ============================================================

//  STATE VARIABLES

// ============================================================

bool scanReady      = true;

unsigned long lastScanTime = 0;

String lastCardID   = "";

unsigned long unlockStartTime = 0;

bool isUnlocking    = false;      // solenoid temporarily open

bool lastRockerState = LOW;       // for debounce

// ============================================================

//  SETUP

// ============================================================

void setup() {

Serial.begin(9600);

rdmSerial.begin(9600);

pinMode(RELAY_PIN,      OUTPUT);

pinMode(BUZZER_PIN,     OUTPUT);

pinMode(LED_GREEN,      OUTPUT);

pinMode(LED_RED,        OUTPUT);

pinMode(ROCKER_SWITCH,  INPUT);

// Start with solenoid LOCKED

digitalWrite(RELAY_PIN, RELAY_LOCK_STATE);

digitalWrite(BUZZER_PIN, LOW);

digitalWrite(LED_GREEN, LOW);

digitalWrite(LED_RED,   LOW);

Wire.begin();

lcd.init();

lcd.backlight();

delay(200);

lcdPrint(" Cabinet System ", "  Initializing  ");

delay(2000);

startupSignal();

Serial.println("==================================");

Serial.println("   RFID Solenoid Locker v2        ");

Serial.println("   Rocker OFF = disabled          ");

Serial.println("   Rocker ON  = enabled (locked)  ");

Serial.println("   Tap any card → open 5 sec      ");

Serial.println("==================================");

updateDisplayAndLock();

}

// ============================================================

//  MAIN LOOP

// ============================================================

void loop() {

unsigned long now = millis();

// ----- Auto-relock after timeout -----

if (isUnlocking && (now - unlockStartTime >= UNLOCK_TIME)) {

digitalWrite(RELAY_PIN, RELAY_LOCK_STATE);

isUnlocking = false;

digitalWrite(LED_GREEN, LOW);

digitalWrite(LED_RED,   HIGH);   // back to locked = red solid

lcdPrint("   LOCKED       ", "Tap Card to Open");

Serial.println("Solenoid relocked (timeout)");

}

// ----- Rocker switch reading with debounce -----

bool rockerOn = digitalRead(ROCKER_SWITCH) == HIGH;

if (rockerOn != lastRockerState) {

delay(50);   // debounce

rockerOn = digitalRead(ROCKER_SWITCH) == HIGH;

if (rockerOn != lastRockerState) {

lastRockerState = rockerOn;

updateDisplayAndLock();

}

}

// ----- System disabled -----

if (!rockerOn) {

// Force solenoid locked, ignore card reader

if (!isUnlocking) {

digitalWrite(RELAY_PIN, RELAY_LOCK_STATE);

}

digitalWrite(LED_GREEN, LOW);

digitalWrite(LED_RED,   LOW);

lcdPrint(" SYSTEM OFF    ", "Switch to Enable");

flushRDM();

delay(100);

return;

}

// ----- System enabled (rocker ON) -----

// If not currently unlocking, show locked state and ensure solenoid locked

if (!isUnlocking) {

lcdPrint("   LOCKED       ", "Tap Card to Open");

digitalWrite(LED_GREEN, LOW);

digitalWrite(LED_RED,   HIGH);      // solid red = locked

digitalWrite(RELAY_PIN, RELAY_LOCK_STATE);

}

// ----- Reset scan cooldown -----

if (!scanReady && (now - lastScanTime >= SCAN_COOLDOWN)) {

scanReady  = true;

lastCardID = "";

flushRDM();

Serial.println("Ready for next scan...");

}

if (!scanReady) {

flushRDM();

return;

}

// ----- Read RFID card -----

String cardID = readRDM6300();

if (cardID == "") return;

if (cardID == lastCardID) return;   // same card still present

scanReady    = false;

lastScanTime = now;

lastCardID   = cardID;

Serial.print("Card UID: ");

Serial.println(cardID);

// ----- Unlock the solenoid (if not already unlocking) -----

if (!isUnlocking) {

digitalWrite(RELAY_PIN, RELAY_UNLOCK_STATE);

isUnlocking = true;

unlockStartTime = now;

digitalWrite(LED_GREEN, HIGH);

digitalWrite(LED_RED,   LOW);

beep(300);

lcdPrint("   OPENING      ", "  Take your item ");

Serial.println("Solenoid UNLOCKED for 5 seconds");

} else {

// Card tapped while already open – just beep

beep(100);

lcdPrint("   ALREADY OPEN ", "  Close manually ");

delay(800);

// restore locked display if not unlocking

if (!isUnlocking) {

lcdPrint("   LOCKED       ", "Tap Card to Open");

}

}

}

// ============================================================

//  UPDATE DISPLAY & LOCK STATE ACCORDING TO ROCKER SWITCH

// ============================================================

void updateDisplayAndLock() {

bool rockerOn = digitalRead(ROCKER_SWITCH) == HIGH;

if (!rockerOn) {

digitalWrite(RELAY_PIN, RELAY_LOCK_STATE);

digitalWrite(LED_GREEN, LOW);

digitalWrite(LED_RED,   LOW);

lcdPrint(" SYSTEM OFF    ", "Switch to Enable");

} else {

digitalWrite(RELAY_PIN, RELAY_LOCK_STATE);

digitalWrite(LED_GREEN, LOW);

digitalWrite(LED_RED,   HIGH);    // solid red = locked

lcdPrint("   LOCKED       ", "Tap Card to Open");

}

}

// ============================================================

//  RDM6300 READ FUNCTION (no changes needed)

// ============================================================

String readRDM6300() {

if (rdmSerial.available() < RDM_PACKET_LEN) return "";

byte firstByte = rdmSerial.read();

if (firstByte != RDM_STX) {

flushRDM();

return "";

}

byte buf[RDM_PACKET_LEN];

buf[0] = RDM_STX;

for (int i = 1; i < RDM_PACKET_LEN; i++) {

unsigned long timeout = millis();

while (!rdmSerial.available()) {

if (millis() - timeout > 100) return "";

}

buf[i] = rdmSerial.read();

}

if (buf[RDM_PACKET_LEN - 1] != RDM_ETX) {

flushRDM();

return "";

}

byte checksum = 0;

for (int i = 1; i <= 9; i += 2) {

checksum ^= hexCharsToByte(buf[i], buf[i + 1]);

}

byte receivedChecksum = hexCharsToByte(buf[11], buf[12]);

if (checksum != receivedChecksum) return "";

String cardID = "";

for (int i = 1; i <= 10; i++) cardID += (char)buf[i];

return cardID;

}

byte hexCharsToByte(byte high, byte low) {

byte h = (high >= 'A') ? (high - 'A' + 10) : (high - '0');

byte l = (low  >= 'A') ? (low  - 'A' + 10) : (low  - '0');

return (h << 4) | l;

}

void flushRDM() {

while (rdmSerial.available()) rdmSerial.read();

}

void lcdPrint(String line1, String line2) {

lcd.clear();

lcd.setCursor(0, 0);

lcd.print(line1);

lcd.setCursor(0, 1);

lcd.print(line2);

}

void beep(int duration_ms) {

digitalWrite(BUZZER_PIN, HIGH);

delay(duration_ms);

digitalWrite(BUZZER_PIN, LOW);

}

void startupSignal() {

for (byte i = 0; i < 2; i++) {

digitalWrite(LED_GREEN, HIGH);

beep(200);

delay(100);

digitalWrite(LED_GREEN, LOW);

delay(200);

}

delay(300);

}